Remove NV for BLE stack

- Drop the provided patch file inside of the SDK（For TuyaOS BLE SDK V3.12.1+, just do this is ok）.

- Open the common/Startup/osal_icall_ble.c file and comment out the osal_snv_init( ); line in the stack_main function

After making these changes, the basic_ble project was able to compile and run with no issues. To update the memory allocation view, you will need to remove the references to NVS_SIZE, NVS_SLOT, and NVS_BASE from the .cmd file.

These modifications will not be needed in the 9.10 SDK as R&D will incorporate them into the SDK release and provide an easy-to-enable option for this.

ADV Random Delay

Open the common/Startup/osal_icall_ble.c file and comment out the USE_HSM conditional statement